Nuclear fission

The process of splitting an atomic nucleus into two smaller nuclei and the release of two and three neutrons and energy

Chain reaction

- Each reaction causes more reactions which cause more reactions

- Energy is released in a nuclear reactor as a result of the nuclear fission and this then produces a chain reaction of fission events

Fission neutrons

- When a nucleus undergoes fission (a neutron was shot to a nucleus), it releases:

- Two or three fission neutrons at high speed

- Radiation (Energy)

- Kinetic energy of the fission neutrons and the fragment nuclei

Fissionable isotopes

- The fuel in a nuclear reactor must contain fissionable isotopes

- Mostly 238-Uranium

– non fissionable but will change into heavy elements including 239-Plutionium, which is fissionable

- 2-3% 235-Uranium – fissionable

Inside a nuclear reactor

- moderator

- atoms in water molecules slow down the fission neutrons by collisions

- Coolant

- Water molecules gain kinetic energy from the neutrons and the fuel rods

- control rods

- absorb surplus neutrons and keep the chain under control

- the depth of the control rods in the core is adjusted to maintain a steady chain reaction

- reactor core

- made of thick steel to withstand the high temperature and energy

- enclosed by thick concrete walls

- absorb radiation from the reactor

Nuclear fusion

The process of forcing two atomic nuclei close together enough so they from a single larger nucleus

Fusion reactions

- Making two light nuclei collide at very high speed and temperature to overcome the force of repulsion

- the nuclei approaching each other will repel each other due to their positive charges

- release energy only if the relative mass of the nucleus formed is no more than 55 (about the same with iron)

- energy was carried away as kinetic energy

Fusion reactors

- the plasma is heated by passing a very large electric current through it

- the plasma is contained by a magnetic field so it doesn’t touch the reactor walls (If it did, it would go cold and the fusion would stop)

| FUSION REACTOR | FISSION REACTOR |
| --- | --- |
| Fuel for fusion reactor is readily available (heavy hydrogen is naturally present in sea water) | Fuel for fission reactor is uranium, which is only found in a certain part of the world |
| The product, helium is a non radioactive inert gas and hence it is harmless | The waste product has to be stored securely for many years as they are radioactive |
| The energy release could be used to generate electricity | The energy release could be used to generate electricity |
| STILL UNDER DEVELOPMENT | HAVE BEEN OPERATED FOR OVER 50 YEARS |